TENTA ELECTRIC INDUSTRIAL CO., LTD.

MAIN INTERFERENCE SUPRESSION CAPACITOR CLASS  X2……MEX--X2

#### SPECIFICATION:

- REFERENCE STANDARDS：

SEV    DIN EN 132 400(1994)/IEC 384-14(1993) AND IEC DRAFT 40 (SEC)677

NEMKO DIN EN 132 400(1994)/IEC 384-14(1993) AND IEC DRAFT 40 (SEC)677

DEMKO DIN EN 132 400(1994)/IEC 384-14(1993) AND IEC DRAFT 40 (SEC)677

SEMKO DIN EN 132 400(1994)/IEC 384-14(1993) AND IEC DRAFT 40 (SEC)677

FIMKO DIN EN 132 400(1994)/IEC 384-14(1993) AND IEC DRAFT 40 (SEC)677

VDE   IEC 60384-14: 1993-07, Table II + A1:1995-06

EK   APPLIED STANDARD: K60384-14 (CAPACITANCE AREA 0.1 to1.0uF)

CQC  THE STANDARDS FOR THE PRODUCTS TO BE TESTED: GBIT14472-1998

CAN/CSA—C22.2 NO.1-94(250V AC)

UL 1414 & UL 1283(250V AC)

USED FOR ACROSS-THE-LINE CAPACITORS,

ANDTENNA-COUPLINE,

LINE-BY-PASS COMPONENTS,

RADIO INTERFERENCE SUPPRESSION,

ELECTRONMAGNETIC INTERFERENCE FILTERS.

- RATED VOLTAGE:

275V AC	 50/ 60 Hz

- CAPACITANCE RANGE:

0.001µF to 2.2 µF

- CAPACITANCE TOLERANCE:

± 5% (J)  ± 10% (K)  ± 20% (M)

- DIELECTRIC:

METALLIZED  POLYPROPYLENE  FLIM

- DISSIPATION FACTOR Tan δ:

LESS THAN 0.1% AT 1 kHz  AND  20℃

- INSULATION RESISTANCE:

BETWEEN ELECTRODES

- LESS THAN OR EAUAL TO 0.33μF≧ 30,000 MΩ

- GREATER THAN 0.33μF≧ 10,000 MΩ/μF

MEASURED AT 100VDC, 60 SEC. AND 20℃

- WITHSTAND VOLTAGE：

APPLIED 1183V.DC FOR 60 SEC. OR 2000V.DC FOR 1 SEC.

- CLIMATE CATEGORY: IN ACCORDANCE WITH DIN40040 GMF

G (MINIMUM LIMIT TEMPERATURE)= -40℃

M (MAXIMUM LIMIT TEMPERATURE)= +100℃

F (HUMIDITY CATEGORY)=AVERAGE RELATEIVE HVMIDITY ≦75%, 95% FOR 30

DAYS PER YEAR, CONTINUOUSLY; 85% FOR THE

REMATINING DAYS, OCCASIONALLY

- HUMIDITY TEST:

TEST CONDIDITON

TEST TEMPER ATURE    : 40±2℃

RELATIVE HUMIDITY    : 90 TO 95%

TEST DURATION         : 500 HRS

TEST CRITERIA

- CAPACITANCE DRIFT     : ≦ 5% OF THE INITIAL VALUE

- INSULATION RESISTANCE : ≦ 50% OF SPECIFIED VALUE

- LIFE TEST:

TEST CONDITION

TEST TEMPERATURE    : 100±3℃

TEST VOLTAGE         : 440VAC AND 1000VAC 60HZ FOR A PERIOD OF

0.1 SEC. ONCE EACH HOUR

TEST DURATION        : 1008 HRS

TEST CRITERIA

- CAPACITANCE DRIFT     : ≦3% OF THE INITIAL VALUE

- INSULATION RESISTANCE : ≧50% OF SPECIFIED VALUE

- VISUAL INSPECTION     : NO DETECTABLE DAMAGE

- SOLDERABILITY:

TEST CONDITIONS

SOLDER BATH TEMPERATURE : 260±5℃

SOLDER                     : sn/cu=99.3%/0.7%

SOLDER TIME                : 5±1 SEC

TEST CRITERIA

75% OF THE SURFACE TINNING

- VIBRATION RESISTANCE: (JIS 5115)

TEST CONDITIONS

FREQUENCY CYCLE : 1 MINUTE PER CYCLE 10-55-10 HZ

TEST DURATION    : PERPENDICULAR DIRECTION WITH THE SAME

METHOD FOR 45 MINUTES EACH, TOTAL 90 MINUTES

TEST CRITERIA

(1) APPERANCE     : NO VISUAL DAMAGE

(2) CONTACT        : NORMAL

- TENSILE STRENGTH OF ELECTRODES:

TEST CONDITIONS

LOAD CONDITIONS  : 1.0KGS

HOLDING TIME      : 10±1 SEC

TEST CRITERIA

NO WIRE BROKEN AND NO DAMAGE OF THE CAPACITOR

- MARKING:

CAPACITORS ARE MARKED WITH TYPE IDENTIFICATION, CAPACITANCE,

CAPACITANCE TOLERANCE, VOLTAGE AND APPROVED CERTIFICATION MARK

- APPROVED BY: SEV: NEMKO: DEMKO; SEMKO; FIMKO; VDE;CQC;CSA;UL;EK

SEV     FILE NO. 99,7 70170,01

NEMKO  FILE NO. P99100418

DEMKO  FILE NO. DK99-01936

SEMKO  FILE NO. 9921015/01

FIMKO   FILE NO. 208003

VDE     FILE NO. 119119

CQC    FILE NO. CQC03001003039

UL      FILE NO. E186475

CSA     FILE NO. LR 109774-1

EK     FILE NO. SU03019-5001

| Normal Size Size |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAP | W | H | T | P | dØ | CAP | W | H | T | P | dØ |
| 0.001 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.068 | 18.0 | 12.0 | 6.0 | 15.0 | 0.8 |
| 0.0012 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.082 | 18.0 | 12.0 | 6.0 | 15.0 | 0.8 |
| 0.0015 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.1 | 18.0 | 13.5 | 6.0 | 15.0 | 0.8 |
| 0.0018 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.12 | 18.0 | 13.5 | 7.5 | 15.0 | 0.8 |
| 0.0022 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.15 | 18.0 | 14.5 | 8.5 | 15.0 | 0.8 |
| 0.0027 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.15 | 26.5 | 15.0 | 6.0 | 22.5 | 0.8 |
| 0.0033 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.18 | 26.5 | 16.5 | 7.0 | 22.5 | 0.8 |
| 0.0039 | 13.0 | 9.0 | 4.0 | 10.0 | 0.6 | 0.22 | 26.5 | 16.5 | 7.0 | 22.5 | 0.8 |
| 0.0047 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.27 | 26.5 | 17.0 | 8.5 | 22.5 | 0.8 |
| 0.0056 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.33 | 26.5 | 17.0 | 8.5 | 22.5 | 0.8 |
| 0.0068 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.39 | 26.5 | 19.0 | 10.0 | 22.5 | 0.8 |
| 0.0082 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.47 | 26.5 | 19.0 | 10.0 | 22.5 | 0.8 |
| 0.01 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.47 | 32.0 | 20.0 | 11.0 | 27.5 | 0.8 |
| 0.012 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.56 | 32.0 | 20.0 | 11.0 | 27.5 | 0.8 |
| 0.015 | 13.0 | 11.0 | 5.0 | 10.0 | 0.6 | 0.68 | 32.0 | 20.0 | 11.0 | 27.5 | 0.8 |
| 0.018 | 13.0 | 12.0 | 6.0 | 10.0 | 0.6 | 0.82 | 32.0 | 22.0 | 13.0 | 27.5 | 0.8 |
| 0.022 | 13.0 | 12.0 | 6.0 | 10.0 | 0.6 | 1.0 | 32.0 | 22.0 | 13.0 | 27.5 | 0.8 |
| 0.027 | 13.0 | 12.0 | 6.0 | 10.0 | 0.6 | 1.0 | 37.0 | 25.0 | 13.5 | 31.5 | 0.8 |
| 0.033 | 18.0 | 11.0 | 5.0 | 15.0 | 0.8 | 1.2 | 37.0 | 25.0 | 13.5 | 31.5 | 0.8 |
| 0.039 | 18.0 | 11.0 | 5.0 | 15.0 | 0.8 | 1.5 | 37.0 | 26.0 | 15.0 | 31.5 | 0.8 |
| 0.047 | 18.0 | 11.0 | 5.0 | 15.0 | 0.8 | 1.8 | 38.0 | 26.0 | 17.0 | 31.5 | 0.8 |
| 0.056 | 18.0 | 12.0 | 6.0 | 15.0 | 0.8 | 2.2 | 37.0 | 29.0 | 19.0 | 31.5 | 0.8 |
| Compact Size ze |  |  |  |  |  |  |  |  |  |  |  |
| 0.1 | 18.0 | 12.0 | 6.0 | 15.0 | 0.8 | 0.33 | 26.5 | 16.5 | 7.0 | 22.5 | 0.8 |
| 0.15 | 18.0 | 13.5 | 7.5 | 15.0 | 0.8 | 0.47 | 26.5 | 17.0 | 8.5 | 22.5 | 0.8 |
| 0.18 | 18.0 | 14.5 | 8.5 | 15.0 | 0.8 | 0.56 | 26.5 | 19.0 | 10.0 | 22.5 | 0.8 |
| 0.22 | 18.0 | 14.5 | 8.5 | 15.0 | 0.8 | 0.68 | 26.5 | 19.0 | 10.0 | 22.5 | 0.8 |
| 0.33 | 18.0 | 16.0 | 10.0 | 15.0 | 0.8 | 1.0 | 32.0 | 20.0 | 11.0 | 27.5 | 0.8 |

*Please contact us for special item or size not listed

| µF / V | Tolerance | W | H | T | P | dØ |
| --- | --- | --- | --- | --- | --- | --- |
| 105K/275VAC |  | 26.5 | 19 | 10 | 22.5 | 0.8 |