AMBE-1000™ Vocoder Chip

-- Not Recommended For New Designs -- 
Please refer to the AMBE-2000/2020 product line!

Introduction

Digital Voice System’s AMBE-1000™ Vocoder Chip* is an extremely flexible high performance vocoder. The AMBE-1000™ Vocoder Chip is a low cost, DSP based full-duplex voice coding solution for real-time and non-real time voice compression applications.

The AMBE-1000™ Vocoder Chip uses DVSI’s proprietary AMBE® Voice Compression Technology to deliver excellent voice quality even at low data rates. Proven to be a superior alternative to linear predictive model vocoders, the AMBE® Voice Compression Technology has been chosen as the standard for several major satellite and mobile radio communications systems.

The AMBE-1000™ Vocoder Chip includes a number of advanced features such as an automatic Voice/Silence detection (VAD), adaptive comfort noise generation, DTMF detection and signaling, echo cancellation and low power saving modes.

Operation

rates from 2400 to 9600 bps. The total rate is the combination of the speech data and the Forward Error Correction (FEC). The AMBE-1000™ has the flexibility to change the speech and FEC rates in 50 bps increments. High FEC rates enable the vocoder to maintain acceptable speech quality even at BER rates as high as 5%. This is a feature that is ideal for mobile communication applications that experience degraded channel conditions and/or significant bit errors.

The AMBE-1000™ Vocoder Chip is easily set-up to transmit and receive digitized speech to and from most linear, A-law, or m-law A/D-D/A (codec) chips through a serial interface. Even programmable A/D-D/A codecs can be configured through a single interface with a controlling processor. The digitized speech from the external A/D chip is converted into compressed digital data (encoded) by the AMBE-1000™ Vocoder Chip and output to the channel interface. The channel interface is typically a processor, controller, modem or similar device that handles the transmission of data across the channel. Simultaneously, the AMBE-1000™ Vocoder Chip receives compressed digital data from the other side of the channel. This received data is reconstructed into a digital speech signal (decoded) by the AMBE-1000™ Vocoder Chip and then sent to the D/A where it is converted back into an analog signal. The encoder and decoder functions are fully asynchronous. The interface to the channel can be configured as serial or parallel interface to accommodate many low cost micro-processor solutions

Features

- DVSI's Full Duplex AMBE® Voice Coder

- Supports Data Rates of 2.4 kbps to 9.6 kbps with User Selectable Error Correction

- DTMF

- Echo Cancellation

- Self-Test Loop-Back Mode

- Voice Activation/ Noise Insertion

- Selectable Serial or Parallel Channel Interface

- A/D-D/A Serial Interface can accept A-law, mu-Law, and Linear data

- Superior Voice Quality

- Variable Data Rates - 2.4 kbps to 9.6 kbps

- Robust to Bit Errors & Background Noise

- Low Cost

- Very Low Power

- Compact Single I.C. Solution

|  |  |  |  | Supported Bit Rates:. Data Rates; 2400 to 9600 bps. FEC Rates; 0 to 4800 bps. Physical Properties:. Package Type; 100 pin TQFP. Operating Temperature; -40°C to +85°C. Storage Temperature; -65°C to +150°C. Interface:. Internal Clock Frequency; 26 - 30 MHz. Input Clock Type; Crystal. I/O Levels; CMOS. Power Usage:. Supply Voltage; 5 volts; 3 volts. Normal Processing; 180mW; 65mW. Standard Sleep Mode; 36mW; 13mW. Hardware and Software Stop Mode; 55mW; 0.11mW. |
| --- | --- | --- | --- | --- |
|  |  |  | *Not recommended for new designs. Please refer to new AMBE-2000/2020™ product line. |  |
|  |  |  | Download the AMBE-1000 Manual |  |
| Digital Voice Systems, Inc. 234 Littleton Road Westford, MA 01886 USA Tel: 978 392 0002 Fax: 978 392 8866 info@dvsinc.com |  |  |  |  |
| Copyright © 2006 Digital Voice Systems, Inc. All Rights Reserved. |  |  |  |  |